| BLOOD CULTURE TIP SHEET |  |
| --- | --- |
| - Remove caps from culture bottles and disinfect culture bottles green tube top with alcohol pad. - Optimal blood draw volume is 20-22cc. 10cc per culture bottle. ***If unable to obtain optimal blood volume, fill (in this order): aerobic (blue) bottle completely (3 mL minimum), then remainder, if any, in anaerobic (purple) bottle. - Make sure to put site on label. Do not cover bar code label on bottles. - Do not cover the window with the measuring device on the side (Lab needs to be able to ensure proper blood amounts are in the bottles. - Place labels VERTICALLY instead of around the bottle (Lab needs to be able to scan the barcode). - If using the pneumatic tube system, place green top in a separate bag towards the necks of the bottles. Place Each bottle in a separate sealed zip lock bag and then securely double bag the set. Do not tube more than one set per tube |  |
| PERIPHERAL STICKS ONLY w/NO CENTRAL LINE | CENTRAL LINE + PERIPHERAL STICK OR - CENTRAL LINE DRAWS |
| - Green tops not needed - Scrub proposed site of venipuncture with ChloraPrep™ for 30 seconds using frictional back and forth motion. Allow to air dry naturally for 30 seconds prior to venipuncture. - DO NOT re-palpate prepared site due to introducing contamination into the culture. - Cultures obtained from the same site must be at least 10 minutes apart; from different sites they may be obtained at the same time. - *PRINTING LABELS – 3 labels print out for each culture set whether or not utilizing green tops - Pick 2 labels from the 1st three that print, discard the third label: label purple & blue bottles. - Pick 2 labels from the 2nd three that print, discard the third label: label the purple & blue bottles. - Ensure each culture set has matching numbers (19RL #) | - One Green top per site (1 for peripheral and 1 for central) - Remove current central line cap from line. Scrub the open port with alcohol. Attach new cap to lumen using proper sterile technique. - Disinfect new central line cap with alcohol for 30 seconds. Wait 30 seconds for alcohol to dry. - Attach 10-20cc luer lock syringe to new cap. Aspirate blood filling whole syringe. DO NOT FLUSH line prior to attaching syringe and do not waste any blood. - Attach red access transfer device to blood-filled syringe and transfer blood. Allow blood to flow passively into each bottle. Do not force blood into bottles! - Wait at least 10 minutes before obtaining second cultures set from the same line. Make sure to print new stickers at timing of second draw. - Ensure each culture set has matching numbers (19RL #) |