# DECANTING

CHEMISTRY RANDOM URINES (VMACR, VCRE, UPHOS etc.) and ARUP SENDOUTS

- Go to General Lab to receive your specimen

- Go to SMART and Select Processing - Decant

- Scan or type the CID and select WDDEC in the drop down for the spot. Select Get CID

Special instructions such as ARUP testing guidelines will appear here

- Select Processed and your aliquot label will print.

(Select Cancel at the next pop up to ignore the rack processing prompt.)

**This label goes on the aliquot tube**

URINE DRUG SCREEN (WDAUP, WDTOXP)

- Follow the same process as above

- DAU’s will pop up with a mandatory Result field; type in DONE and select SAVE. Your labels will now print.

These aliquot labels will both go on the yellow urine tubes

**If you are Decanting a WDUTOXP you will also receive a label that goes on the sendout aliquot tube**

24HR URINE PROCESSING

- Go to General Lab to receive your specimen

2. In the Order Codes box add UDATA to all 24hr. specimens. Select Save

3. Another box will pop up

- In the text field Enter; ”date and time started” TO “date and time ended” .

- Select SAVE

- Go to SMART and Select PROCESSING - DECANT

- Scan or type the CID and select WDDEC in the drop down for the spot. Select Get CID

Enter Total Volume in result field

Select Save

Select Processed

Labels will print that will be used for the aliquot tube

***IF YOU ARE ORDERING THE 24HOUR URINE TESTS YOU CAN ALSO ENTER THE UDATA IN ORDER ENTRY***